APRS COMMAND · QUICK START APRS COMMAND Quick Start Guide v1.0 ON THE AIR IN 10 MINUTES Install · Set Up · Watch the Map — the fast path James Rospopo · KE4CON August 05, 2026

# Read This First (Really — It's 60 Seconds)

This guide gets you from download to a live map of nearby ham radio activity as fast as possible. It skips the details on purpose — the full User Manual has everything else. Five short steps and you're running.

You will NOT transmit anything. APRS Command starts in listen-only mode. The title bar shows a TX Disabled badge, and nothing you do in this guide puts a signal on the air. So relax and click around — you can't key up the radio by accident.

Use a practice callsign while you learn. Type N0CALL wherever a callsign is asked for. It's the standard "not a real station" callsign for testing, and it lets you explore everything without pretending to be a licensed station. Swap in your real callsign only when you're ready to actually operate.

# Step 1 — Install It

1.	Go to the official downloads page: github.com/KE4CON/APRS-Command/releases.

2.	Grab the file for your computer and run it:

▪	Windows: the …-windows-x64-Setup.exe installer — double-click it.

▪	Mac: the …-macos-….dmg — open it and drag APRS Command into Applications.

▪	Linux / Raspberry Pi: the .deb or .rpm package.

If you see a security warning, that's normal. Depending on the build, Windows or macOS may show a one-time "are you sure?" caution the first time. It's safe — just click through it. (The User Manual's Installing chapter explains exactly what to click for each.)

SCREENSHOT The GitHub releases page with the Windows, macOS, and Linux download files

# Step 2 — Fill In the Welcome Screen

The first time it opens, a small setup window appears. Fill in five things — that's all it wants:

1.	Callsign: type N0CALL (your practice callsign).

2.	Latitude and Longitude: your approximate location in decimal degrees (e.g. 42.33 and -88.45). A quick web search of your town plus "lat long" works fine.

3.	Distance units: pick Miles or Kilometers.

4.	Receive filter radius: how far out to pull in activity — 125 is a good start.

5.	Click Save and continue.

None of this is permanent. You can change any of it later under Settings → Station. Get it roughly right and move on.

SCREENSHOT The First-Run Setup window with Callsign, Latitude, Longitude, units, and radius filled in

# Step 3 — Meet the Map (and the Safety Badge)

You land on the map — the main screen. Glance at the top-right corner and you'll see two small badges:

▪	TX Disabled — your proof that nothing is transmitting. Good. Leave it alone.

▪	APRS-IS Offline — you're not connected to a data feed yet. That's the next step, and it's why the map is empty right now.

SCREENSHOT The map page just after setup: title bar with the APRS-IS Offline and TX Disabled badges, empty map

# Step 4 — Fill the Map With Real Activity

An empty map is just a map with no data yet. The fastest way to see real, live stations is to connect to APRS-IS — the internet side of the APRS network. No radio required.

1.	On the menu bar, click Settings, then the Connections tab.

2.	From the type dropdown, choose APRS-IS and click Add.

3.	Fill in the boxes on the right:

▪	Server: rotate.aprs2.net

▪	Port: 14580

▪	Passcode: -1  (this means receive-only — no transmitting)

▪	Filter: leave blank (it uses your location automatically)

1.	Make sure the port's Enabled box is checked, then click Save.

Within a few seconds the badge flips to APRS-IS Online and stations start appearing on the map. That's it — you're live.

The -1 passcode keeps you receive-only. With the passcode set to -1, you're pulling in a full live feed but sending nothing. A real passcode is only needed if you ever choose to transmit — not now.

SCREENSHOT The Connections tab with an APRS-IS port set to rotate.aprs2.net / 14580 / passcode -1, and stations populating the map

# Step 5 — Drive the Map

You're running. Here's everything you need to get around:

▪	Move: click and drag the map.

▪	Zoom: roll the mouse wheel — it zooms toward the pointer.

▪	Who's that?  click any station marker to see its details in the bottom-left.

▪	Change the map: use the Base map dropdown in the top-left corner (streets, topo, or aerial).

▪	Home button: the ⌂ icon on the left edge snaps back to your area if you get lost.

# That's It — Now What?

You've got a live APRS map in about ten minutes. Everything below is optional and fully explained in the User Manual whenever you want it:

▪	Messages, weather radar, and the station list — under the View, Map, and Messages menus.

▪	Draw on the map (search areas, coverage, notes) — Map → Drawing Toolbar.

▪	Practice safely with Replay, Simulation, and Training — none of it touches the air.

▪	When you're ready to transmit — set up your real callsign and turn on transmit, deliberately, in Settings. The manual walks you through it.

Want the whole picture? The full User Manual covers every feature, step by step, in the same plain language. This guide was just the fast lane.

Welcome aboard, and 73 (that's ham for "best regards").